02R96 MIDI SPECIFICATIONS

Ver 1.00

Revision History

| Version | Date | Description |
| --- | --- | --- |
| Ver 1.00 | Apr. 2002 | For 02R96 V1.0 release |

Table of Contents

1.	DATA FORMAT	1

1.1	CHANNEL MESSAGE	1

1.2	SYSTEM COMMON MESSAGE	1

1.3	SYSTEM REALTIME MESSAGE	1

1.4	EXCLUSIVE MESSAGE	1

1.4.1	Real Time System Exclusive	1

1.4.2	System Exclusive Message	1

1.4.2.1	Bulk Dump	1

1.4.2.2	PARAMTER CHANGE	1

2.	Format Details	3

2.1	NOTE OFF  (8n)	3

2.2	NOTE ON   (9n)	3

2.3	CONTROL CHANGE (Bn)	3

2.4	PROGRAM CHANGE (Cn)	4

2.5	SONG POSITION POINTER (F2)	4

2.6	TIMING CLOCK (F8)	4

2.7	START  (FA)	4

2.8	CONTINUE (FB)	4

2.9	STOP  (FC)	5

2.10	ACTIVE SENSING (FE)	5

2.11	SYSTEM RESET (FF)	5

2.12	SYSTEM EXCLUSIVE MESSAGE (F0)	6

2.12.1	MIDI MACHINE CONTROL (MMC)	6

2.12.2	BULK DUMP	6

2.12.2.1	Scene memory bulk dump format	7

2.12.2.2	Scene memory bulk dump request format	7

2.12.2.3	Setup memory bulk dump format	8

2.12.2.4	Setup memory bulk dump request format	8

2.12.2.5	User define layer bulk dump format	9

2.12.2.6	User define layer bulk dump request format	9

2.12.2.7	User define plug-in bulk dump format	10

2.12.2.8	User define plug-in bulk dump request format	10

2.12.2.9	User Define Key bulk dump format	11

2.12.2.10	User Define Key bulk dump request format	11

2.12.2.11	Control change table bulk dump format	12

2.12.2.12	Control change table bulk dump request format	12

2.12.2.13	Program change table bulk dump format	13

2.12.2.14	Program change table bulk dump request format	13

2.12.2.15	Equalizer library bulk dump format	14

2.12.2.16	Equalizer library bulk dump request format	14

2.12.2.17	Compressor library bulk dump format	15

2.12.2.18	Compressor library bulk dump request format	15

2.12.2.19	Gate library bulk dump format	16

2.12.2.20	Gate library bulk dump request format	16

2.12.2.21	Effect library bulk dump format	17

2.12.2.22	Effect library bulk dump request format	17

2.12.2.23	Channel library bulk dump format	18

2.12.2.24	Channel library bulk dump request format	18

2.12.2.25	Input patch library bulk dump format	19

2.12.2.26	Input patch library bulk dump request format	19

2.12.2.27	Output patch library bulk dump format	20

2.12.2.28	Output patch library bulk dump request format	20

2.12.2.29	Bus to Stereo library bulk dump format	21

2.12.2.30	Bus to Stereo library bulk dump request format	21

2.12.2.31	Surround Monitor library bulk dump format	22

2.12.2.32	Surround Monitor library bulk dump request format	22

2.12.2.33	Automix bulk dump format (Compress)	23

2.12.2.34	Automix bulk dump request format (Compress)	23

2.12.2.35	Plug-in effect card bulk dump format	24

2.12.2.36	Plug-in effect card bulk dump request format	24

2.12.3	PARAMETER CHANGE	25

2.12.3.1	Parameter change basic format (02R96 only)	25

2.12.3.2	Parameter Change basic format (Universal format)	25

2.12.3.3	Parameter request basic format (02R96 only)	26

2.12.3.4	Parameter request basic format (Universal format)	26

2.12.3.5	Parameter Address	26

2.12.3.6	Parameter change (Edit buffer)	27

2.12.3.7	Parameter request (Edit buffer)	27

2.12.3.8	Parameter change (Patch data)	28

2.12.3.9	Parameter request (Patch data)	28

2.12.3.10	Parameter change (Setup memory)	29

2.12.3.11	Parameter request (Setup memory)	29

2.12.3.12	Parameter change  (Backup memory)	30

2.12.3.13	Parameter request (Backup memory)	30

2.12.3.14	Parameter change (Function call Library: store / recall)	31

2.12.3.15	Parameter change (Function call: title)	32

2.12.3.16	Parameter request (Function call: title)	32

2.12.3.17	Parameter change (Function call: Scene/Library Clear)	33

2.12.3.18	Parameter change (Function call: pair, copy)	34

2.12.3.19	Parameter change (Function call Event: Effect )	35

2.12.3.20	Parameter change (Key remote)	36

2.12.3.21	Parameter change (Remote Meter)	37

2.12.3.22	Parameter request (Remote Meter)	37

2.12.3.23	Parameter change (Remote Time Counter)	38

2.12.3.24	Parameter request (Remote Time Counter)	38

2.12.3.25	Parameter change (Automix Status)	39

2.12.3.26	Parameter request (Automix Status)	39

# MESSAGE TYPE

## CHANNEL MESSAGE

| Command | rx/tx | function |
| --- | --- | --- |
| 8n NOTE OFF | rx | Control the internal effects |
| 9n NOTE ON | rx | Control the internal effects |
| Bn CONTROL CHANGE | rx/tx | Control parameters |
| Cn PROGRAM CHANGE | rx/tx | Switch scene memories |

## SYSTEM COMMON MESSAGE

| Command | rx/tx | function |
| --- | --- | --- |
| F1 MIDI TIME CODE QUARTER FRAME | rx | Used when TIME REFERENCE is MIDI CLOCK. |
| F2 SONG POSITION POINTER | rx | Used when TIME REFERENCE is MIDI CLOCK. |

## SYSTEM REALTIME MESSAGE

| Command | rx/tx | function |
| --- | --- | --- |
| F8 TIMING CLOCK | rx | MIDI clock |
| FA START | rx* | Start automix (from the beginning) |
| FB CONTINUE | rx* | Start automix (from the middle) |
| FC STOP | rx* | Stop automix |
| FE ACTIVE SENSING | rx | Check MIDI cable connections |
| FF RESET | rx | Clear running status |

* Received only when the Automix TIME REFERENCE setting is MIDI CLOCK.

## EXCLUSIVE MESSAGE

### Real Time System Exclusive

| Command | rx/tx | function |
| --- | --- | --- |
| F0 7F dd 06 MMC COMMAND | tx | MMC command (refer to MMC specification) |
| F0 7F dd 07 MMC RESPONSE | rx | MMC response (refer to MMC specification) |
| F0 7F dd 01 MIDI TIME CODE | rx | Used when TIME REFERENCE is MTC. |

### System Exclusive Message

#### Bulk Dump

| Command | rx/tx | function |
| --- | --- | --- |
| F0 43 0n 7E BULK DUMP DATA | rx/tx | BULK DUMP DATA |
| F0 43 2n 7E BULK DUMP REQUEST | rx/tx | BULK DUMP REQUEST |

The following data types of bulk dump are used on the 02R96.

| Data name | tx/rx | function |
| --- | --- | --- |
| ‘m’ | tx/rx* | Scene Memory & Request |
| ‘S’ | tx/rx | Setup Memory & Request |
| ‘a’ | tx/rx* | Automix data & Request |
| ‘R’ | tx/rx | Input patch library & Request |
| ‘O’ | tx/rx | Output patch library & Request |
| ‘H’ | tx/rx* | Channel library & Request |
| ‘G’ | tx/rx* | Gate library & Request |
| ‘Y’ | tx/rx* | Compressor library & Request |
| ‘Q’ | tx/rx* | Equalizer library & Request |
| ‘E’ | tx/rx* | Effect library & Request |
| ‘J’ | tx/rx* | Bus to Stereo library & Request |
| ‘K’ | tx/rx* | Surround Monitor library & Request |
| ‘P’ | tx/rx* | Program change table & Request |
| ‘C’ | tx/rx | Control change table & Request |
| ‘L’ | tx/rx* | User define layer & Request |
| ‘I’ | tx/rx* | Plug-in User define & Request |
| ‘V’ | tx/rx | User define key & Request |
| ‘N’ | tx/rx* | Plug-in Effect Card Data & Request |

* The bulk dump for DM2000 is also receivable.

#### PARAMTER CHANGE

| Command | rx/tx | function |
| --- | --- | --- |
| F0 43 1n 3E 0B PARAMETER CHANGE | rx/tx | 02R96-specific parameter change |
| F0 43 3n 3E 0B PARAMETER REQUEST | rx/tx | 02R96-specific parameter request |
| F0 43 1n 3E 7F PARAMETER CHANGE | rx/tx | General purpose digital mixer parameter change |
| F0 43 3n 3E 7F PARAMETER REQUEST | rx/tx | General purpose digital mixer parameter request |

The following data types of parameter change are used by the 02R96.

| Type | tx/rx | Function |
| --- | --- | --- |
| 1 | tx/rx | Edit buffer |
| 2 | tx/rx | Patch data |
| 3 | tx/rx | Setup data |
| 4 | tx/rx | Backup data |
| 16 | tx/rx | Function (recall, store, title) |
| 17 | rx | Function (pair) |
| 18 | rx | Function (event) |
| 32 | rx | Key remote |
| 33 | tx/rx | remote meter |
| 34 | tx/rx | remote counter |
| 35 | tx/rx | Automix status |

** tx means that the data can be transmitted from the 02R96. rx means that the data can be received by the 02R96.

# Format Details

## NOTE OFF		(8n)

<< Reception >>

Received when the [Rx CH] matches.

Used to control effects.

|  | STATUS | 1000nnnn | 8n | Note off message |  |
| --- | --- | --- | --- | --- | --- |
|  | DATA | 0nnnnnnn | nn | Note number |  |
|  |  | 0vvvvvvv | vv | Velocity(ignored) |  |

## NOTE ON			(9n)

<< Reception >>

Received when the [Rx CH] matches.

Used to control effects.

|  | STATUS | 1001nnnn | 9n | Note on message |  |
| --- | --- | --- | --- | --- | --- |
|  | DATA | 0nnnnnnn | nn | Note number |  |
|  |  | 0vvvvvvv | vv | Velocity(1-127:on, 0:off) |  |

## CONTROL CHANGE	(Bn)

<< Reception >>

Received when [Control Change Rx] is ON and the [Rx CH] matches. However if [OMNI] is ON, this is received regardless of the channel. If [Control Change ECHO] is ON, these messages are echoed to MIDI OUT. If [TABLE] is selected, parameters will be controlled according to the set-tings of the [Control assign table]. The parameters that can be set are de-fined in the CONTROL CHANGE ASSIGN PARAMETER LIST. If [NRPN] is selected, four messages are used to control the defined parameter: NRPN control numbers (62h, 63h) and DATA ENTRY control numbers (06h, 26h). Parameter settings are defined in the CONTROL CHANGE ASSIGN PARAMETER LIST.

<< Transmission >>

If [TABLE] is selected, operating the parameters specified in the [Control assign table] will cause these messages to be transmitted on the [Tx CH] if [Control Change TX] is ON. The parameters that can be specified are defined in the CONTROL CHANGE ASSIGN PARAMETER LIST. If [NRPN] is selected, operating the specified parameters will cause data to be transmitted on the [Tx CH] if [Control Change TX] is ON, using four messages: NRPN control numbers (62h, 63h) and DATA ENTRY control numbers (06h, 26h). Parameter settings are defined in the CONTROL CHANGE ASSIGN PARAMETER LIST.

If [ TABLE ] is selected

|  | STATUS | 1011nnnn | Bn | Control change |  |
| --- | --- | --- | --- | --- | --- |
|  | DATA | 0nnnnnnn | nn | Control number (0-95, 102-119) |  |
|  |  | 0vvvvvvv | vv | Control Value (0-127) |  |

If [ NRPN ] is selected

|  | STATUS | 1011nnnn | Bn | Control change |  |
| --- | --- | --- | --- | --- | --- |
|  | DATA | 01100010 | 62 | NRPN LSB |  |
|  |  | 0vvvvvvv | vv | LSB of parameter number |  |
|  | STATUS | 1011nnnn | Bn | Control change | *1 |
|  | DATA | 01100011 | 63 | NRPN MSB |  |
|  |  | 0vvvvvvv | vv | MSB of parameter number |  |
|  | STATUS | 1011nnnn | Bn | Control change | *1 |
|  | DATA | 00100110 | 26 | LSB of data entry |  |
|  |  | 0vvvvvvv | vv | LSB of parameter data |  |
|  | STATUS | 1011nnnn | Bn | Control change | *1,*2 |
|  | DATA | 00000110 | 06 | MSB of data entry | *2 |
|  |  | 0vvvvvvv | vv | MSB of parameter data | *2 |

*1) There is no particular need to add the 2nd, 3rd, and 4th status　for transmission. However if these are present during reception, they should be received.

*2) Does not need to be transmitted if the parameter data fits within 7 bits.

## PROGRAM CHANGE	(Cn)

<< Reception >>

This message is received if [Program Change RX] is ON and [Rx CH] matches. However if [OMNI] is ON, this is received regardless of the channel.

A scene memory will be recalled according to the settings of the [Program Change Table].

This message will be echoed if [Program Change ECHO] is ON.

<< Transmission >>

If [Program Change TX] is ON, this message is transmitted according to the settings of the [Program Change Table] on the [Tx CH] channel when a scene memory is recalled.

If the recalled scene has been assigned to more than one program number, the lowest-numbered program number will be transmitted. Transmission to Studio Manager using Program Change messages will not be performed since there is no guarantee that the contents of the tables will match. (Parameter Changes will always be used.)

|  | STATUS | 1100nnnn | Cn | Program change |  |
| --- | --- | --- | --- | --- | --- |
|  | DATA | 0nnnnnnn | nn | Program number (0-127) |  |

## SONG POSITION POINTER	(F2)

<< Reception >>

If this is received when the automix TIME REFERENCE setting is MIDI CLOCK, the automix will move to the song position that was received.

|  | STATUS | 11110010 | F2 | Song position pointer |  |
| --- | --- | --- | --- | --- | --- |
|  | DATA | 0vvvvvvv | vv | Song position LSB |  |
|  |  | 0vvvvvvv | vv | Song position MSB |  |

## TIMING CLOCK	(F8)

<< Reception >>

If the automix TIME REFERENCE setting is MIDI CLOCK, this message is used to synchronize automix. It is also used to control effects. This message is transmitted 24 times per quarter note.

| STATUS | 11111000 | F8 | Timing clock |
| --- | --- | --- | --- |

## START		(FA)

<< Reception >>

This message is received if the automix TIME REFERENCE setting is MIDI CLOCK, and will start the automix. In actuality, automix will start when the next TIMING CLOCK is received after receiving the START message.

| STATUS | 11111010 | FA | Start |
| --- | --- | --- | --- |

## CONTINUE	(FB)

<< Reception >>

This message is received if the automix TIME REFERENCE setting is MIDI CLOCK, and will cause automix to start from the current song position. In actuality, automix will start when the next TIMING CLOCK is received after receiving the CONTINUE message.

| STATUS | 11111011 | FB | Continue |
| --- | --- | --- | --- |

## STOP		(FC)

<< Reception >>

This message is received if the automix TIME REFERENCE setting is MIDI CLOCK, and will cause automix to stop.

| STATUS | 11111100 | FC | Stop |
| --- | --- | --- | --- |

## ACTIVE SENSING	(FE)

<< Reception >>

Once this message has been received, the failure to receive any message for an interval of 400 ms or longer will cause MIDI transmission to be initialized, such as by clearing the Running Status.

| STATUS | 11111101 | FE | Active sensing |
| --- | --- | --- | --- |

## SYSTEM RESET	(FF)

<< Reception >>

When this message is received, MIDI communications will be cleared, e.g., by clearing the Running Status.

| STATUS | 11111111 | FF | System reset |
| --- | --- | --- | --- |

## SYSTEM EXCLUSIVE MESSAGE	(F0)

### MIDI MACHINE CONTROL	(MMC)

These messages are transmitted when the Machine Control section of the 02R96 is operated. For details, refer to the MMC specification.

### BULK DUMP

This message sends or receives the contents of various memories stored within the 02R96.

The basic format is as follows.

###### For DUMP DATA

F0 43 0n 7E cc cc <Model ID> tt mm mm [Data …] cs F7

###### For DUMP REQUEST

F0 43 2n 7E <Model ID> tt mm mm F7

| n | Device Number |
| --- | --- |
| cc cc | DATA COUNT (the number of bytes that follow this, ending before the checksum) |
| <Model ID> | Model ID (for the 02R96, this is 4C 4D 20 20 38 43 35 34) |
| tt | DATA TYPE |
| mm mm | DATA NUMBER |
| cs | CHECK SUM |

A unique header (Model ID) is used to determine whether the device is a 02R96.

CHECK SUM is obtained by adding the bytes that follow BYTE COUNT (LOW) and end before CHECK SUM, taking the binary compliment of this sum, and then setting bit 7 to 0.

CHECK SUM = (-sum)&0x7F

<< Reception >>

This message is received if [Bulk RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

When a bulk dump is received, it is immediately written into the specified memory.

When a bulk dump request is received, a bulk dump is immediately transmitted.

About some types, The bulk dump for DM2000 is also receivable.

<< Transmission >>

This message is transmitted on the [Tx CH] by key operations in the [MIDI]-[BULK DUMP] screen.

A bulk dump is transmitted on the [Rx CH] in response to a bulk dump request.

The data area is handled by converting seven words of 8-bit data into eight words of 7-bit data.

[Conversion from actual data into bulk data]

d[0~6]: actual data

b[0~7]: BULK data

b[0] = 0;

for( I=0; I<7; I++){

if( d[I]&0x80){

b[0] |= 1<<(6-I);

}

b[I+1] = d[I]&0x7F;

}

[Restoration from bulk data into actual data]

d[0~6]: actual data

b[0~7]: BULK data

for( I=0; I<7; I++){

b[0] <<= 1;

d[I] = b[I+1]+(0x80&b[0]);

}

#### Scene memory bulk dump format

The 02R96 can transmit and receive scene memories in compressed form.

The second and third bytes of the DATA NAME indicate the scene number.

0:Scene no.0 – 99:Scene no.99, 256:Edit buffer

For reception by the 02R96, only the user area is valid. (1-99, 256)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01101101 | 6D | ‘m’ |
|  |  | 0mmmmmmm | mh | m=0-99, 256(Scene0-99, EDIT BUFFER) |
|  |  | 0mmmmmmm | ml | Receive is effective 1-99, 256 |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Scene data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Scene memory bulk dump request format

The second and third bytes of the DATA NAME indicate the scene number that is being requested. If this is 256, the data of the edit buffer will be bulk-dumped.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01101101 | 6D | ‘m’ |
|  |  | 0mmmmmmm | mh | m=0-99, 256(Scene0-99, EDIT BUFFER) |
|  |  | 0mmmmmmm | ml | Receive is effective 1-99, 256 |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Setup memory bulk dump format

Of the setup memory of the 02R96, this bulk-dumps data other than the User define layer, User define plug-in, User define keys, Control change table, and Program change table.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010011 | 53 | ‘S’ |
|  |  | 00000010 | 02 |  |
|  |  | 00000000 | 00 | No.256 = Current |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Setup data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Setup memory bulk dump request format

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010011 | 53 | ‘S’ |
|  |  | 00000010 | 02 |  |
|  |  | 00000000 | 00 | No.256 = Current |
|  | EOX | 11110111 | F7 | End of exclusive |

#### User define layer bulk dump format

The second and third bytes of the DATA NAME indicate the bank number.

Be aware that the state of the transmission destination will (in some cases) change if the same bank is being used.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001100 | 4C | ‘L’ |
|  |  | 00000000 | 00 |  |
|  |  | 0bbbbbbb | bb | b=0-3(bank no.1-4) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | User define layer data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### User define layer bulk dump request format

The second and third bytes of the DATA NAME indicate the bank number.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001100 | 4C | ‘L’ |
|  |  | 00000000 | 00 |  |
|  |  | 0bbbbbbb | bb | b=0-3(bank no.1-4) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### User define plug-in bulk dump format

The second and third bytes of the DATA NAME indicate the bank number.

Be aware that the state of the transmission destination will (in some cases) change if the same bank is being used.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001001 | 49 | ‘I’ |
|  |  | 00000000 | 00 |  |
|  |  | 0bbbbbbb | bb | b=0-7(bank no.1-8) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | User define plug-in data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### User define plug-in bulk dump request format

The second and third bytes of the DATA NAME indicate the bank number.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001001 | 49 | ‘I’ |
|  |  | 00000000 | 00 |  |
|  |  | 0bbbbbbb | bb | b=0-7(bank no.1-8) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### User Define Key bulk dump format

The second and third bytes of the DATA NAME indicate the bank number.

Be aware that the state of the transmission destination will (in some cases) change if the same bank is being used.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010110 | 56 | ‘V’ |
|  |  | 00000000 | 00 |  |
|  |  | 0bbbbbbb | bb | b=0-3(bank no.A-D) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | User define key data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### User Define Key bulk dump request format

The second and third bytes of the DATA NAME indicate the bank number.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010110 | 56 | ‘V’ |
|  |  | 00000000 | 00 |  |
|  |  | 0bbbbbbb | bb | b=0-3(bank no.A-D) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Control change table bulk dump format

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01000011 | 43 | ‘C’ |
|  |  | 00000010 | 02 |  |
|  |  | 00000000 | 00 | No.256 = Current |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Control change table data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Control change table bulk dump request format

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01000011 | 43 | ‘C’ |
|  |  | 00000010 | 02 |  |
|  |  | 00000000 | 00 | No.256 = Current |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Program change table bulk dump format

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010000 | 50 | ‘P’ |
|  |  | 00000010 | 02 |  |
|  |  | 00000000 | 00 | No.256 = Current |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Program change table data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Program change table bulk dump request format

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010000 | 50 | ‘P’ |
|  |  | 00000010 | 02 |  |
|  |  | 00000000 | 00 | No.256 = Current |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Equalizer library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.1 - 199:Library no.200, 256:CH1 - 311:CH56, 384:BUS1 - 391:BUS8, 512:AUX1 - 519:AUX8, 768:STEREO L - 769:STEREO R

256 and following are data for the corresponding channel of the edit buffer.

For reception by the 02R96, only the user area is valid. (40-199, 256-)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010001 | 51 | ‘Q’ |
|  |  | 0mmmmmmm | mh | 0-199(EQ Library no.1-200), |
|  |  | 0mmmmmmm | ml | 256-(Channel current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | EQ Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Equalizer library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010001 | 51 | ‘Q’ |
|  |  | 0mmmmmmm | mh | 0-199(EQ Library no.1-200), |
|  |  | 0mmmmmmm | ml | 256-(Channel current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Compressor library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.1 - 127:Library no.128, 256:CH1 - 311:CH56, 384:BUS1 - 391:BUS8, 512:AUX1 - 519:AUX8, 768:STEREO L - 769:STEREO R

256 and following are data for the corresponding channel of the edit buffer.

For reception by the 02R96, only the user area is valid. (36-127, 256-)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01011001 | 59 | ‘Y’ |
|  |  | 0mmmmmmm | mh | 0-127(COMP Library no.1-128), |
|  |  | 0mmmmmmm | ml | 256-(Channel current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | COMP Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Compressor library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01011001 | 59 | ‘Y’ |
|  |  | 0mmmmmmm | mh | 0-127(COMP Library no.1-128), |
|  |  | 0mmmmmmm | ml | 256-(Channel current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Gate library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.1 - 127:Library no.128, 256:CH1 - 311:CH56

256 and following are data for the corresponding channel of the edit buffer.

For reception by the 02R96, only the user area is valid. (4-127, 256-)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01000111 | 47 | ‘G’ |
|  |  | 0mmmmmmm | mh | 0-127(GATE Library no.1-128), |
|  |  | 0mmmmmmm | ml | 256-311(Channel current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | GATE Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Gate library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01000111 | 47 | ‘G’ |
|  |  | 0mmmmmmm | mh | 0-127(GATE Library no.1-128), |
|  |  | 0mmmmmmm | ml | 256-311(Channel current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Effect library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.1 - 127:Library no.128, 256:EFFECT1 - 259:EFFECT4

256-259 are the data for the corresponding area of the edit buffer.

For reception by the 02R96, only the user area is valid. (52-127, 256-259)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01000101 | 45 | ‘E’ |
|  |  | 0mmmmmmm | mh | 0-127(Effect Library no.1-128), |
|  |  | 0mmmmmmm | ml | 256-259(Effect1-4 current) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Effect Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Effect library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01000101 | 45 | ‘E’ |
|  |  | 0mmmmmmm | mh | 0-127(Effect Library no.1-128), |
|  |  | 0mmmmmmm | ml | 256-259(Effect1-4 current) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Channel library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.0 - 128:Library no.128, 256:CH1 - 311:CH56, 384:BUS1 - 391:BUS8, 512:AUX1 - 519:AUX8, 768:STEREO L - 769:STEREO R

256 and following are the data for the corresponding channel of the edit buffer.

For reception by the 02R96, only the user area is valid. (2-128, 256-)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001000 | 48 | ‘H’ |
|  |  | 0mmmmmmm | mh | 0-128(Channel Library no.0-128), |
|  |  | 0mmmmmmm | ml | 256-(Current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Channel Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Channel library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001000 | 48 | ‘H’ |
|  |  | 0mmmmmmm | mh | 0-128(Channel Library no.0-128), |
|  |  | 0mmmmmmm | ml | 256-(Current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Input patch library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.0 - 32:Library no.32, 256:current input patch data

For reception by the 02R96, only the user area is valid. (1-32, 256)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010010 | 52 | ‘R’ |
|  |  | 0mmmmmmm | mh | 0-32(Input patch Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Input patch Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Input patch library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01010010 | 52 | ‘R’ |
|  |  | 0mmmmmmm | mh | 0-32(Input patch Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Output patch library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.0 - 32:Library no.32, 256:current output patch data

For reception by the 02R96, only the user area is valid. (1-32, 256)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001111 | 4F | ‘O’ |
|  |  | 0mmmmmmm | mh | 0-32(Output patch Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Output patch Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Output patch library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001111 | 4F | ‘O’ |
|  |  | 0mmmmmmm | mh | 0-32(Output patch Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Bus to Stereo library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.0 - 32:Library no.32, 256:current data

For reception by the 02R96, only the user area is valid. (1-32,256)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001010 | 4A | ‘J’ |
|  |  | 0mmmmmmm | mh | 0-32(Bus to stereo Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Bus to stereo Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Bus to Stereo library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001010 | 4A | ‘J’ |
|  |  | 0mmmmmmm | mh | 0-32(Bus to stereo Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Surround Monitor library bulk dump format

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.0 - 32:Library no.32, 256:current data

For reception by the 02R96, only the user area is valid. (1-32, 256)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001011 | 4B | ‘K’ |
|  |  | 0mmmmmmm | mh | 0-32(Surround Monitor Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | BLOCK INFO. | 0ttttttt | tt | total block number(minimum number is 0) |
|  |  | 0bbbbbbb | bb | current block number(0-total block number) |
|  | DATA | 0ddddddd | ds | Surround Monitor Library data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Surround Monitor library bulk dump request format

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001011 | 4B | ‘K’ |
|  |  | 0mmmmmmm | mh | 0-32(Surround Monitor Library no.0-32), |
|  |  | 0mmmmmmm | ml | 256(Current data) |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Automix bulk dump format (Compress)

The second and third bytes of the DATA NAME indicate the library number.

0:Library no.1 - 15:Library no.16, 256:current automix data

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01100001 | 61 | ‘a’ |
|  |  | 0mmmmmmm | mh | 0-15(Automix no.1-16), 256(Current data) |
|  |  | 0mmmmmmm | ml |  |
|  | BLOCK INFO. | 0bbbbbbb | bh | current block number(0-total block number) |
|  |  | 0bbbbbbb | bl |  |
|  |  | 0ttttttt | th | total block number(minimum number is 0) |
|  |  | 0ttttttt | tl |  |
|  | DATA | 0ddddddd | ds | Automix memory data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Automix bulk dump request format (Compress)

The second and third bytes of the DATA NAME indicate the library number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01100001 | 61 | ‘a’ |
|  |  | 0mmmmmmm | mh | 0-15(Automix no.1-16), 256(Current data) |
|  |  | 0mmmmmmm | ml | 0 |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Plug-in effect card bulk dump format

The second and third bytes of the DATA NAME indicate the slot number.

0:SLOT 3 - 2:SLOT 4

The data is not received if the Developer ID and Product ID are different than the card that is installed in the slot.

The data is not transmitted if a valid plug-in effect card is not installed.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0000nnnn | 0n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  | COUNT HIGH | 0ccccccc | ch | data count = ch * 128 + cl |
|  | COUNT LOW | 0ccccccc | cl |  |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001110 | 4E | ‘N’ |
|  |  | 0mmmmmmm | mh | 0-1(SLOT 3-4) |
|  |  | 0mmmmmmm | ml |  |
|  | BLOCK INFO. | 0bbbbbbb | bh | current block number(0-total block number) |
|  |  | 0bbbbbbb | bl |  |
|  |  | 0ttttttt | th | total block number(minimum number is 0) |
|  |  | 0ttttttt | tl |  |
|  |  | 0000iiii | 0i | Developer id (High) |
|  |  | 0000iiii | 0i | Developer id (Low) |
|  |  | 0000jjjj | 0j | Product id (High) |
|  |  | 0000jjjj | 0j | Product id (Low) |
|  | DATA | 0ddddddd | ds | Plug-in Effect card memory data of block[bb] |
|  |  | : | : |  |
|  |  | 0ddddddd | de |  |
|  | CHECK SUM | 0eeeeeee | ee | ee=(Invert(‘L’+…+de)+1)&0x7F |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Plug-in effect card bulk dump request format

The second and third bytes of the DATA NAME indicate the slot number. (See above)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0010nnnn | 2n | n=0-15 (Device number=MIDI Channel) |
|  | FORMAT No. | 01111110 | 7E | Universal bulk dump |
|  |  | 01001100 | 4C | ‘L’ |
|  |  | 01001101 | 4D | ‘M’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00100000 | 20 | ‘ ’ |
|  |  | 00111000 | 38 | ‘8’ |
|  |  | 01000011 | 43 | ‘C’ |
|  |  | 00110101 | 35 | ‘5’ |
|  |  | 00110100 | 34 | ‘4’ |
|  | DATA NAME | 01001110 | 4E | ‘N’ |
|  |  | 0mmmmmmm | mh | 0-1(SLOT 3-4) |
|  |  | 0mmmmmmm | ml | 0 |
|  | EOX | 11110111 | F7 | End of exclusive |

### PARAMETER CHANGE

#### Parameter change basic format (02R96 only)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 0ttttttt | tt | Data type |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | DATA *) | 0ddddddd | dd | data |
|  |  | : | : |  |
|  | EOX | 11110111 | F7 | End of exclusive |

*) For parameters with a data size of 2 or more, data for that size will be transmitted.

#### Parameter Change basic format (Universal format)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 0ttttttt | tt | Data type |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | DATA *) | 0ddddddd | dd | data |
|  |  | : | : |  |
|  | EOX | 11110111 | F7 | End of exclusive |

*) For parameters with a data size of 2 or more, data for that size will be transmitted.

#### Parameter request basic format (02R96 only)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 0ttttttt | tt | Data type |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter request basic format (Universal format)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 0ttttttt | tt | Data type |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter Address

Consult your dealer for parameter address details.

#### Parameter change	(Edit buffer)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the specified parameter will be controlled.

<<Transmission>>

If [Parameter change TX] is ON and a parameter not specified in the [Control assign table] is modified, this message will be transmitted with the device number specified by the [Tx CH].

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00000001 | 01 | Edit Buffer |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | DATA | 0ddddddd | dd | data |
|  |  | : | : |  |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter request	(Edit buffer)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the value of the specified parameter will be transmitted as a Parameter Change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00000001 | 01 | Edit Buffer |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change	(Patch data)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the specified parameter will be controlled.

<<Transmission>>

If [Parameter change TX] is ON and a parameter belonging to this type is modified, this message will be transmitted with the device number specified by the [Tx CH].

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00000010 | 02 | Patch data |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | DATA | 0ddddddd | dd | data |
|  |  | : | : |  |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter request	(Patch data)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the value of the specified parameter will be transmitted as a Parameter Change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00000010 | 02 | Patch data |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change	(Setup memory)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the value of the specified parameter will be controlled.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00000011 | 03 | Setup data |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | DATA | 0ddddddd | dd | data |
|  |  | : | : |  |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter request	(Setup memory)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the value of the specified parameter will be transmitted as a Parameter Change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00000011 | 03 | Setup data |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change 	(Backup memory)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the value of the specified parameter will be controlled.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00000100 | 04 | Backup data |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | DATA | 0ddddddd | dd | data |
|  |  | : | : |  |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter request	(Backup memory)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the value of the specified parameter will be transmitted as a Parameter Change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00000100 | 04 | Backup data |
|  |  | 0eeeeeee | ee | Element no.(If 'ee' is 0, 'ee' is expanded to two bytes) |
|  |  | 0ppppppp | pp | Parameter no. |
|  |  | 0ccccccc | cc | Channel no. |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change	(Function call Library: store / recall)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the specified memory/library will be stored/recalled.

<<Transmission>>

If [Parameter change ECHO] is ON, this message will be retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00010000 | 10 | Function call |
|  |  | 0fffffff | ff | function |
|  |  | 0mmmmmmm | mh | number High |
|  |  | 0mmmmmmm | ml | number Low |
|  | DATA | 0ccccccc | ch | channel High |
|  |  | 0ccccccc | cl | channel Low |
|  | EOX | 11110111 | F7 | End of exclusive |

| function |  | number | channel*1) | tx/rx |
| --- | --- | --- | --- | --- |
| SCENE RECALL | 0x00 | 0-99 | 256 | tx*2)/rx |
| EQ LIB RECALL | 0x01 | 1-200 | 0-513 | tx/rx |
| GATE LIB RECALL | 0x02 | 1-128 | 0-95 | tx/rx |
| COMP LIB RECALL | 0x03 | 1-128 | 0-513 | tx/rx |
| EFF LIB RECALL | 0x04 | 1-128 | 0-7 | tx/rx |
| CHANNEL LIB RECALL | 0x06 | 0-128 | 0-513 | tx/rx |
| INPATCH LIB RECALL | 0x07 | 0-32 | 256 | tx/rx |
| OUTPATCH LIB RECALL | 0x08 | 0-32 | 256 | tx/rx |
| Bus to Stereo LIB RECALL | 0x09 | 0-32 | 256 | tx/rx |
| Surround Monitor LIB RECALL | 0x0A | 0-32 | 256 | tx/rx |
| AUTOMIX LIB RECALL | 0x0B | 1-16 | 256 | tx/rx |
| SCENE STORE | 0x20 | 1-99 | 256, 16383 | tx/rx |
| EQ LIB STORE | 0x21 | 41-200 | 0-513, 16383 | tx/rx |
| GATE LIB STORE | 0x22 | 5-128 | 0-96, 16383 | tx/rx |
| COMP LIB STORE | 0x23 | 37-128 | 0-513, 16383 | tx/rx |
| EFF LIB STORE | 0x24 | 53-128 | 0-7, 16383 | tx/rx |
| CHANNEL LIB STORE | 0x26 | 3-128 | 0-513, 16383 | tx/rx |
| INPATCH LIB STORE | 0x27 | 1-32 | 256, 16383 | tx/rx |
| OUTPATCH LIB STORE | 0x28 | 1-32 | 256, 16383 | tx/rx |
| Bus to Stereo LIB STORE | 0x29 | 1-32 | 256, 16383 | tx/rx |
| Surround Monitor LIB STORE | 0x2A | 1-32 | 256, 16383 | tx/rx |
| AUTOMIX LIB STORE | 0x2B | 1-32 | 256, 16383 | tx/rx |

*1)	0:CH1 - 55:CH56, 128:BUS1 - 135:BUS8, 256:AUX1 -263:AUX8, 512:STEREO L - 513:STEREO R

Use 256 if the recall destination or store source is a single data item.

Effect is 0:Effect 1 - 3:Effect 4

If the store destination is 16383 (0x3FFF), this indicates that the library data has been changed by a external cause (i.e., receiving from MIDI)

(only transmitted by the 02R96)

*2)	This is also transmitted when a program that has not been assigned to the [Program change table] is recalled.

(Normally this would be transmitted as a program change message.)

#### Parameter change	(Function call: title)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the title of the specified memory/library will be changed.

<<Transmission>>

In response to a request, a Parameter Change message will be transmitted on the [Rx CH].

If [Parameter change ECHO] is ON, this message will be retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00010000 | 10 | Function call |
|  |  | 0100ffff | 4f | title function |
|  |  | 0mmmmmmm | mh | number High |
|  |  | 0mmmmmmm | ml | number Low |
|  | DATA | 0ddddddd | dd | title 1 |
|  |  | : | : | : |
|  |  | 0ddddddd | dd | title x(depend on the library) |
|  | EOX | 11110111 | F7 | End of exclusive |

| function |  | number | size |
| --- | --- | --- | --- |
| SCENE LIB TITLE | 0x40 | 0-99,256(0:response only) | 16 |
| EQ LIB TITLE | 0x41 | 1-200(1-40:response only) | 16 |
| GATE LIB TITLE | 0x42 | 1-128(1-4:response only) | 16 |
| COMP LIB TITLE | 0x43 | 1-128(1-36:response only) | 16 |
| EFF LIB TITLE | 0x44 | 1-128(1-52:response only) | 16 |
| CHANNEL LIB TITLE | 0x46 | 0-128(0-1:response only) | 16 |
| INPATCH LIB TITLE | 0x47 | 0-32(0:response only) | 16 |
| OUTPATCH LIB TITLE | 0x48 | 0-32(0:response only) | 16 |
| Bus to Stereo LIB TITLE | 0x49 | 0-32(0:response only) | 16 |
| Surround Monitor LIB TITLE | 0x4A | 0-32(0:response only) | 16 |
| AUTOMIX LIB TITLE | 0x4B | 1-16 | 16 |

#### Parameter request	(Function call: title)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, a Parameter Change message will be transmitted on the [Rx CH].

Refer to the above table for the Functions and Numbers.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00010000 | 10 | Function call |
|  |  | 0100ffff | 4f | title function |
|  |  | 0mmmmmmm | mh | number High |
|  |  | 0mmmmmmm | ml | number Low |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change	(Function call: Scene/Library Clear)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the specified memory/library will be cleared.

<<Transmission>>

If [Parameter change ECHO] is ON, this message will be retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00010000 | 10 | Function call |
|  |  | 0110ffff | 6f | clear function |
|  |  | 0mmmmmmm | mh | number High |
|  |  | 0mmmmmmm | ml | number Low |
|  | EOX | 11110111 | F7 | End of exclusive |

| function |  | number |
| --- | --- | --- |
| SCENE LIB CLEAR | 0x60 | 1-99 |
| EQ LIB CLEAR | 0x61 | 41-200 |
| GATE LIB CLEAR | 0x62 | 5-128 |
| COMP LIB CLEAR | 0x63 | 37-128 |
| EFF LIB CLEAR | 0x64 | 53-128 |
| CHANNEL LIB CLEAR | 0x66 | 2-128 |
| INPATCH LIB CLEAR | 0x67 | 1-32 |
| OUTPATCH LIB CLEAR | 0x68 | 1-32 |
| Bus to Stereo LIB CLEAR | 0x69 | 1-32 |
| Surround Monitor LIB CLEAR | 0x6A | 1-32 |
| AUTOMIX LIB CLEAR | 0x6B | 1-16 |

#### Parameter change	(Function call: pair, copy)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, pairing will be enabled/disabled for the specified channel. (Items other than PAIR are reserved for future use.)

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00010001 | 11 | Function call Pair |
|  |  | 0000ffff | 0f | function |
|  |  | 0sssssss | sh | Source channel H |
|  |  | 0sssssss | sl | Source channel L |
|  | DATA | 0ddddddd | dh | Destination channel H |
|  |  | 0ddddddd | dl | Destination channel L |
|  | EOX | 11110111 | F7 | End of exclusive |

| function |  | Channel |
| --- | --- | --- |
| PAIR ON with COPY | 0x00 | *1) |
| PAIR ON with RESET BOTH | 0x01 | *1) |
| PAIR OFF | 0x02 | *1) |
| CHANNEL COPY ( Reserve ) | 0x03 | reserved for future use |
| EQ COPY ( Reserve ) | 0x04 | reserved for future use |
| COMP COPY ( Reserve ) | 0x06 | reserved for future use |
| EFFECT COPY ( Reserve ) | 0x07 | reserved for future use |
| GATE STEREO LINK ON ( Reserve ) | 0x09 | reserved for future use |
| GATE STEREO LINK OFF( Reserve ) | 0x0A | reserved for future use |
| COMP STEREO LINK ON ( Reserve ) | 0x0B | reserved for future use |
| COMP STEREO LINK OFF( Reserve ) | 0x0C | reserved for future use |

*1)	0:CH1 – 55:CH56, 128:BUS1 – 135:BUS8, 256:AUX1 – 263:AUX8, 512:STEREO L – 513:STEREO R

Effect is 0:Effect 1 - 3:Effect 4

- In the case of PAIR, you must specify channels for which pairing is possible.

- In the case of PAIR ON with COPY, you must specify Source Channel as the copy source, and Destination Channel as the copy destination.

#### Parameter change	(Function call Event: Effect )

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the corresponding effect's function activates (depending on the effect type).

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 01111111 | 7F | Universal |
|  | ADDRESS | 00010010 | 12 | Function call Effect Event |
|  |  | 0000ffff | 0f | function |
|  |  | 00000000 | 00 |  |
|  |  | 0ppppppp | pp | Release:0, Press:1 |
|  | DATA | 00000000 | 00 |  |
|  |  | 0eeeeeee | ee | Effect number * |
|  | EOX | 11110111 | F7 | End of exclusive |

* Effect1:0 – Effect4:3

| function |  | Channel |
| --- | --- | --- |
| Freeze Play button | 0x00 | 0:Effect1-3:Effect4 |
| Freeze Record button | 0x01 | 0:Effect1-3:Effect4 |
| Auto Pan 5.1 Trigger Button | 0x02 | 0:Effect1-3:Effect4 |
| Auto Pan 5.1 Reset Button | 0x03 | 0:Effect1-3:Effect4 |

- This does not activate when the effect type is different.

#### Parameter change	(Key remote)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the same processing that is executed when the key specified by Address is pressed (released). (Refer to the PARAMETER CHANGE PARAMETER NUMBER LIST.)

<<Transmission>>

If [Parameter Change ECHO] is ON, this message is retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00100000 | 20 | Key remote |
|  |  | 0kkkkkkk | kk | Key address H |
|  |  | 0kkkkkkk | kk | Key address M |
|  |  | 0kkkkkkk | kk | Key address L |
|  | DATA | 0ppppppp | pp | Release:0, Press:1 |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change	(Remote Meter)

When transmission is enabled by receiving a Request of Remote meter, the specified meter information is transmitted every 50 msec for 10 seconds.

When you want to transmit meter information continuously, a Request must be transmitted continuously within every 10 seconds.

<<Reception>>

This is echoed if [Parameter change ECHO] is ON.

<<Transmission>>

When transmission has been enabled by a Request, the parameter specified by Address (see PARAMETER CHANGE PARAMETER NUMBER LIST) will be transmitted on the [Rx CH] channel at 50 msec intervals for a duration of 10 seconds.

Transmission will be disabled if the power is turned off and on again, or if the PORT setting is changed.

If [Parameter Change ECHO] is ON, this message is retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00100001 | 21 | Remote meter |
|  |  | 0mmmmmmm | mm | ADDRESS UL |
|  |  | 0mmmmmmm | mm | ADDRESS LU |
|  |  | 0mmmmmmm | mm | ADDRESS LL |
|  | DATA | 0ddddddd | dd | Data1 H |
|  |  | 0ddddddd | dd | Data1 L |
|  |  | : | : |  |
|  | EOX | 11110111 | F7 | End of exclusive |

* Meter data uses the unmodified DECAY value of the DSP. For the interpretation of the value, refer to the PARAMETER CHANGE PARAMETER NUMBER LIST.

#### Parameter request	(Remote Meter)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the parameter specified by Address (refer to PARAMETER CHANGE PARAMETER NUMBER LIST) will be transmitted on the [Rx CH] at 50 msec intervals for a duration of 10 seconds.

If Address UL= 0x7F is received, transmission of all meter data will be halted immediately (disable).

<<Transmission>>

If [Parameter Change ECHO] is ON, this message is retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00100001 | 21 | Remote meter |
|  |  | 0mmmmmmm | mm | ADDRESS UL |
|  |  | 0mmmmmmm | mm | ADDRESS LU |
|  |  | 0mmmmmmm | mm | ADDRESS LL |
|  |  | 0ccccccc | ch | Count H |
|  |  | 0ccccccc | cl | Count L |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change	(Remote Time Counter)

When transmission is enabled by receiving a Request of Remote Time Counter, the Time Counter data is transmitted every 50 msec for 10 seconds.

When you want to transmit Counter information continuously, a Request must be transmitted within every 10 seconds.

<<Reception>>

This is echoed if [Parameter change ECHO] is ON.

<<Transmission>>

When transmission is enabled by receiving a Request, the Time Counter information is transmitted on [RxCH] channel every 50 msec for 10 seconds.

Transmission will be disabled if the power is turned off and on again, or if the PORT setting is changed.

If [Parameter Change ECHO] is ON, this message is retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00100010 | 22 | Remote Time counter |
|  |  | 0000tttt | 0t | 0:Time code, 1:Measure.Beat.Clock |
|  |  | 0ddddddd | dd | Hour / Measure H |
|  |  | 0ddddddd | dd | Minute / Measure L |
|  | DATA | 0ddddddd | dd | Second / Beat |
|  |  | 0ddddddd | dd | Frame / Clock |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter request	(Remote Time Counter)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When this is received, the Time Counter information is transmitted on the [Rx CH] channel every 50 msec for 10 seconds.

When the second byte of Address is received on 0x7F, data transmission will be halted immediately (disable).

<<Transmission>>

If [Parameter Change ECHO] is ON, this message is retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00100010 | 22 | Remote Time counter |
|  |  | 0ddddddd | dd | 0:Transmission request, 0x7F:Transmission stop request |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter change	(Automix Status)

When transmission is enabled by receiving a Request of Automix status, the Automix Status data is transmitted every second for 10 seconds. When you want to transmit the Automix Status information continuously, the Request must be transmitted continuously minimum within 10 seconds interval. The data is transmitted continuously while the transmission is enabled, even when the Automix Status on the 02R96 has been changed.

<<Reception>>

This is echoed if [Parameter change ECHO] is ON.

<<Transmission>>

When the transmission is set to enable by receiving a Request. The Automix Status data is transmitted on the [Rx CH] channel every second for 10 seconds. The data is transmitted continuously while the transmission is enabled, even when the Automix Status on the 02R96 has been changed.

Transmission will be disabled if the power is turned off and on again, or if the PORT setting is changed.

If [Parameter Change ECHO] is ON, this message is retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0001nnnn | 1n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00100011 | 23 | Automix status |
|  |  | 00000000 | 00 |  |
|  |  | 0000dddd | 0d | Automix status H |
|  |  | 0000dddd | 0d | Automix status L |
|  | EOX | 11110111 | F7 | End of exclusive |

#### Parameter request	(Automix Status)

<<Reception>>

This is received if [Parameter change RX] is ON and the [Rx CH] matches the device number included in the SUB STATUS.

This is echoed if [Parameter change ECHO] is ON.

When the data is received, the Automix Status data is transmitted on the [Rx CH] every second for 10 seconds.

When the second byte of Address is received on 0x7F, data transmission will be halted immediately (disable).

<<Transmission>>

If [Parameter Change ECHO] is ON, this message is retransmitted without change.

|  | STATUS | 11110000 | F0 | System exclusive message |
| --- | --- | --- | --- | --- |
|  | ID No. | 01000011 | 43 | Manufacture’s ID number (YAMAHA) |
|  | SUB STATUS | 0011nnnn | 3n | n=0-15 (Device number=MIDI Channel) |
|  | GROUP ID | 00111110 | 3E | MODEL ID (digital mixer) |
|  | MODEL ID | 00001011 | 0B | 02R96 |
|  | ADDRESS | 00100011 | 23 | Automix status |
|  |  | 0ddddddd | dd | 0:Transmission request, 0x7F:Transmission stop request |
|  | EOX | 11110111 | F7 | End of exclusive |